5007439

SLAM: Localización de robots y construcción simultánea de mapas

SLAM simulator for MatLab

The slam.m program is a simple SLAM simulation written in MatLab.  A mobile robot carries out a square trajectory in an environment with point features at each side of the trajectory, similar to a cloister (see figure below; red points and trajectory are ground truth):

The vehicle is equipped with a point detector whose characteristics (range, precision) can be modified.  Vehicle odometry can also be modified.  You can also try different data association algorithms: the Nearest Neighbour, the Joint Compatibility Branch and Bound, or your own!

- Load slam.m, find where data association is done and try the nearest neighbour (NN) algorithm.

- Complete SINGLES and try it:

You have observations.m observations, and prediction.n predicted features.

For every observation i, check whether it has only one neighbour,

feature, and whether that feature j  has only that one neighbour

observation i.  If so, H(i) = j.

You will need to check the compatibility.ic matrix

for this:

compatibility.ic(i,j) = 1 if observation i is a neighbour of

feature j.

- Include people (configuration.people = 1)  and try SINGLES

- Try Joint Compatibility Branch and Bound (JCBB)